# Where Hybrid and Regenerative Retrofit Pays Off on the World's Largest Machines

### A positioning review of known techniques, ordered by maturity

Aaron Garcia · aaron@garcia.ltd

Status: Review / synthesis — not a research contribution. Public.

## What this paper is, and what it is not

This is a review. It does not report an experiment, a simulation, a prototype, or a new device, and it does not claim to. Its purpose is narrow and honest: to take a class of machines that rarely get compared side by side — the physically largest powered machines in industry — and ask one practical question about each, where does hybrid or regenerative retrofit actually pay off today, and where is it still a research bet?

Every technique discussed here already exists in the literature. Regenerative energy recovery on cyclic-load machinery is a mature, deployed field. Multi-criteria methods for ranking retrofit opportunities are a mature, deployed field. Magnetic gearing is a decades-old research topic. Nothing below is being claimed as novel. The only thing this paper adds is breadth — putting these machine classes on one page — and candour about the gap between what is deployable and what is not. If you have read a recent review of hydraulic-excavator energy regeneration, you already know most of the deployable half of this document; the contribution, such as it is, is the cross-machine framing and the explicit maturity ranking.

I am not a domain expert in drivetrain engineering. This review is written from the outside, leans on primary sources throughout, and flags clearly where a claim is an extrapolation I cannot back with a citation.

## Why the biggest machines are the interesting case

Draglines, bucket-wheel excavators and large hydraulic excavators share a feature that makes them unusually good candidates for hybrid retrofit: large masses accelerated and decelerated on a short, repeating cycle. A walking dragline slews a loaded bucket through roughly 90–180° and back every 30–60 seconds. A large excavator's upper structure does the same. Each deceleration throws away kinetic energy — conventionally as heat in brakes or throttling losses — that a regenerative path can capture and reuse on the next acceleration.

This is not a subtle effect. In the machines where it has been measured, the swing/slew motion is one of the largest single energy sinks in the duty cycle, precisely because the inertia is high and the cycle is frequent. That combination — high inertia, frequent reversal — is the textbook precondition for energy recovery, and it is why the excavator-swing case has been studied so heavily.

Not every giant machine is in this class, and it matters to say so. Tunnel-boring-machine cutterheads and multi-megawatt wind turbines are also enormous, but their drivetrains do not share the cyclic high-inertia-reversal profile that makes swing recovery pay: a TBM cutterhead grinds under near-continuous, quasi-steady frictional torque, and a wind rotor runs continuously in one direction. Swing/brake energy recovery is largely inapplicable to them. They have their own efficiency and reliability questions, but they are a different problem and this review does not fold them into the regenerative-recovery case. (An earlier private ranking that started this work scored TBM cutterheads at the very top for hybrid retrofit; on closer reading that was a mis-modelling of the TBM duty cycle, and it is corrected here.)

## The mature lever: regenerative energy recovery

The deployable answer for cyclic-load machines is regenerative recovery on the swing/slew axis, using either an electrical path (motor-generator plus supercapacitor or battery buffer) or a hydraulic path (accumulator), or a hybrid of both. This is not speculative. The hydraulic-excavator literature reports, across different architectures, baselines and load conditions:

- ~13% energy improvement on a hybrid boom system with ~97.5% regeneration efficiency [ref 4, boom-regeneration study];

- mean regeneration efficiency ~33% and swing energy saving ~9% in an accumulator scheme [ref 2, IJPEM-GT 2019];

- savings on the order of 37–63% for integrated electrical–hydraulic swing systems, the upper figure being against a purely hydraulic no-load baseline [ref 5, electrical–hydraulic swing];

- a comprehensive 2025 review confirming energy regeneration as the central efficiency lever for hydraulic excavators [ref 3, Singh, Kumar & Thakur, 2025].

The spread (roughly 9–63%) is real and worth understanding: it reflects what you measure against. Savings look largest against an unrecuperated hydraulic baseline and smallest against an already-electrified one. The honest takeaway is not a single headline number but a direction: on machines with frequent high-inertia reversals, recovering swing/brake energy delivers double-digit efficiency gains with commercially available components. On the technology-readiness scale this is TRL 8–9 — built, sold, operating.

The load-bearing caveat for this review. These figures are established for hydraulic excavators. I did not find peer-reviewed, quantified regeneration studies specific to walking draglines or bucket-wheel excavators at the same level of rigour. Those machines share the same cyclic high-inertia-reversal profile, so extending recovery to them is a reasonable engineering expectation — but it is an extrapolation, not a measured result, and this paper does not dress it up as one. That gap is itself a useful finding: the excavator case is solved and documented; the larger cyclic machines are an under-published application of the same mature idea.

## A heuristic for ranking opportunities — and its honest limits

The instinct that started this work was to rank machines rather than argue for one. A simple scoring heuristic — rate each machine on feasibility, environmental benefit, efficiency improvement and novelty, and divide by a risk factor — is enough to sort a candidate list on the back of an envelope and see that the high-inertia cyclic machines rise to the top.

It should be said plainly that this is a heuristic, not a method. The formal version of "score alternatives on weighted criteria and rank them" is Multi-Criteria Decision Analysis (MCDA/MCDM), a mature discipline with established, defensible techniques — AHP, TOPSIS, SIMUS and others — that handle criteria weighting, interdependence and sensitivity analysis properly. MCDA has already been applied to exactly this kind of question, including ranking sustainable-mining and equipment-electrification options on criteria such as energy efficiency, technological feasibility and return on investment [MDPI Sustainability, 2025; MDPI Energies, 2025]. A back-of-envelope product of four scores over a risk term has none of that rigour: the weights are implicit, the criteria are not independent, and there is no sensitivity check. It is useful for a first cut and for communicating intuition; it is not a substitute for a proper MCDA and should not be cited as one. Presented as a heuristic, it earns its keep. Presented as a method, it would be weaker than work already in print.

## The speculative frontier: ultra-scale magnetic gearing

A separate idea attaches to the reliability of these drivetrains rather than their efficiency: replace meshing gear teeth with magnetic coupling, so torque transmits across an air gap with no contact, no tooth wear and inherent overload protection. The coaxial magnetic gear — concentric rotors coupled through a ferromagnetic modulator — was introduced by Atallah & Howe in 2001 and demonstrated a simulated torque density above ~100 kNm/m³ [Atallah & Howe, 2001], and it remains an active research area [MDPI Actuators, 2024].

For the largest machines this is a frontier proposal, not a solution, and this review labels it as such:

- Maturity. Multi-megawatt-scale magnetic gearing is at roughly TRL 2–3 — concept and early lab work. The torque densities these machines need are hundreds of times what has been demonstrated; the scaling gap is large and unproven, and it may not close at all.

- A logical mismatch worth stating. If the reliability problem on these drivetrains is dominated by bearing failures, magnetic gearing does not obviously address it: it removes the gear mesh, while the shafts still run on bearings. A magnetic gear also depends on a small, uniform air gap, which is itself sensitive to bearing wear, shaft deflection and eccentricity. It is therefore reasonable to worry that, absent a redesign that also reduces bearing count, magnetic gearing could leave the dominant failure mode untouched or even aggravate it. This is a concern about the fit between the proposed cure and the stated disease, and it needs answering before the idea is worth pursuing at scale.

Magnetic gearing is included here not because it is ready, but because it is the honest opposite pole of the review: the interesting, high-risk end of the spectrum, useful to name so that it is not confused with the deployable end.

## Reliability, split by failure mode

It is worth being precise rather than tribal about magnetic gearing versus the conventional answer, because they address different failure modes:

- Gear-mesh wear — the tribological failure of loaded, lubricated meshing teeth — is a failure mode magnetic gearing genuinely eliminates by design, since torque crosses an air gap with no contact. If mesh wear dominates a given drivetrain, magnetic gearing is a real long-term structural fix (still unproven at this scale, per the previous section).

- Bearing failure — which the reliability data on these machines suggests dominates — is not addressed by magnetic gearing: the shafts still run on bearings, and the magnetic gear's air-gap tolerance is itself sensitive to bearing wear. For this failure mode the mature, deployed levers are unglamorous and well proven: condition monitoring and vibration analysis, redundancy and graceful degradation, better sealing and lubrication.

So the honest reliability answer is conditional. A practitioner asking "how do I cut my drivetrain failures this year" is pointed at condition monitoring and redundancy. A designer asking "how might mesh-wear failures be engineered out a decade from now" has a legitimate reason to watch magnetic gearing — provided the scaling problem closes and the bearing question is answered.

## Where this leaves a practitioner

- Large hydraulic excavators: regenerative swing recovery is deployable now, double-digit efficiency gains, off-the-shelf components. Do it.

- Draglines and bucket-wheel excavators: the same cyclic physics strongly suggests the same opportunity, but it is not yet quantified in the literature. The valuable next step is measurement on these specific machines, not another argument that it should work.

- TBM cutterheads and wind turbines: different duty cycle — swing recovery does not apply. Their efficiency and reliability questions are real but out of scope here, and should not be lumped in with the cyclic case.

- Reliability: buy it with condition monitoring and redundancy today; watch magnetic gearing as a research bet, not a purchase.

## Limitations — what this review is not

This paper reports no original measurement. Its quantified claims are confined to hydraulic-excavator swing systems, where the literature is solid; every extension to larger machines is flagged as extrapolation. The scoring heuristic is explicitly inferior to formal MCDA and is offered only as intuition. The magnetic-gearing section is a statement of a research frontier and its open problems, not evidence that it works. And the review's own novelty is limited: it overlaps existing single-machine reviews, and adds only cross-machine breadth and an explicit maturity ordering. A reader who needs rigour on any one machine class should go to the primary sources cited here.

## References

1. K. Atallah and D. Howe, "A novel high-performance magnetic gear," IEEE Transactions on Magnetics, 37(4):2844–2846, 2001. doi:10.1109/20.951324

2. "Energy Regeneration and Reuse of Excavator Swing System with Hydraulic Accumulator," International Journal of Precision Engineering and Manufacturing-Green Technology, 2019. doi:10.1007/s40684-019-00157-7

3. S. Singh, A. Kumar, S. Thakur, "Sustainable energy solutions for hydraulic excavators: A comprehensive review and novel energy regeneration approach," Proc. IMechE Part E: J. Process Mechanical Engineering, 2025. doi:10.1177/09544089251324561

4. "Optimization of energy regeneration of hybrid hydraulic excavator boom system." Cited by title; full publication details unverified at time of writing — reader should confirm before relying on the ~13% / ~97.5% figures.

5. "Design and control of a novel hybrid drive swing system for excavators integrating electrical and hydraulic energy recovery systems," Energy (Elsevier), 2025. PII: S036054422504143X.

6. "Energy saving of hybrid hydraulic excavator with innovative powertrain," Energy Conversion and Management, 2021. doi:10.1016/j.enconman.2021.114447

7. "Multicriteria Decision-Making for Sustainable Mining: Evaluating the Transition to Net-Zero-Carbon Energy Systems," Sustainability 17(10):4566, 2025.

8. "Industry Perspectives on Electrifying Heavy Equipment: Trends, Challenges, and Opportunities," Energies 18:2806, 2025. doi:10.3390/en18112806

9. "Optimal Design of a Coaxial Magnetic Gear Considering Thermal Demagnetization and Structural Robustness for Torque Density Enhancement," Actuators 15(1):59, 2024.

License: CC BY-SA 4.0. This document proposes nothing built, funded, reviewed, or contacted; no person or organisation has been approached in connection with it. Where it cites a maturity level (TRL) or an energy figure, that classification is the author's reading of the cited public source.